AUXNAV STUDY QUESTIONS
COMDTPUB P16798.16A


7-2.
 (True/False) Tide data for the reference stations is based on an 18.6 year moving average of mean low water or mean lower low water.

7-3.
 Datum for the east coast of the United States is based on  Mean Lower Low Water.
a. Mean Low Water
b. Mean High Water

c. Mean Lower Low Water
d. Mean Higher High Water
7-4.
  Spring tides occur when the sun and moon are in  alignment and have the greatest tidal ranges.
a. Neap, quadrature
b. Neap, alignment

c. Spring, quadrature
d. Spring, alignment
7-5.
  Neap tides occur when the sun and moon are in quadrature and have the smallest tidal ranges.

a. Neap, quadrature
b. Neap, alignment

c. Spring, quadrature
d. Spring, alignment

7-6.
 Diurnal tides occur on the Gulf coast of the United States and have only one high water and one low water in each lunar day.

a. Mixed
b. Compound

c. Diurnal
d. Semidiurnal

7-8.
height of tide is the difference in depth of water from  datum at any given time.

a. height of tide, datum
b. height of tide, lower low water

c. sea level, height of tide
d. depth of water, datum

7-9.
depth of water is  datum plus the height of tide at any given time.

a. height of tide, datum
b. height of tide, lower low water

c. sea level, height of tide
d. depth of water, datum
7-10.
 datum is the charted  depth of water at any point.

a. depth, datum
b. datum, depth of water
c. height of tide, depth
d. datum, height of tide

7-11.
 (True/False) Diurnal tides occur on the east and west coasts of the United States and have two highs and two lows in each lunar day.

7-12.
 (True/False) Mixed tides are a variation of semidiurnal tides in which the lows are fairly close in height while the highs are far apart.

7-13.
 If the longitude of a location in the western hemisphere is less than the longitude of the standard (time) meridian of the time zone, the location is  earlier of the time meridian and sunrise or sunset is  east than at the time meridian.

a. west, earlier
b. west, later

c. east, later
d. east, earlier
7-14.
 If the longitude of a location in the eastern hemisphere is greater than the longitude of the standard (time) meridian of the time zone, the location is east of the time meridian and sunrise or sunset is  earlier than at the time meridian.
a. west, earlier
b. west, later

c. east, later
d. east, earlier
7-15.
Range of tide at a given location is the difference between adjacent high and low waters at that location.

a. Depth of water
b. Range of tide
c. Height of tide
d. Datum

7-16.
 (True/False) Duration of rise or fall is the time for the tide to ebb, starting at high water, or flood, starting at low water.

7-17.
 (True/False) An in-coming tide is ebbing.

7-18.
 (True/False) An out-going tide is ebbing.

PROBLEMS

7-1.
 What time is sunrise at Little Harbor at Woods Hole, on 1 April (EST is in effect)? (use tables in Appendix A)  0524.
7-2.
 What is the maximum range of tide at Newport, Rhode Island on 6 April?  3.4 ft.
.
7-3.
 What is the maximum duration of tide at Newport on 7 April? 6 hr 35 min.
7-4.
 What is the height of tide at Tarpaulin Cove at 1506 on 6 Apr. (EDT is in effect)?

(Use tables in Appendix A) 0.8 ft.
7-5.
 What time is sunset at Tarpaulin Cove on 6 April? 1814
7-6.
 What is the depth of water at Little Harbor at 1100 on 2 April if the charted depth is 24 feet?

 24.6 ft.
AUXNAV: Ch7_Questions.doc
Updated:  February 8, 2005
page 1

